# 1) What is STP (Spanning Tree Protocol)?

STP (Spanning Tree Protocol) is a Layer 2 protocol used in switches to prevent network loops.

### Simple Definition

STP prevents loops by blocking extra paths and keeping only one safe path active.

# 2) Why Does a Loop Get Created?

A loop happens when multiple switches are connected with more than one path (redundant links).

Here:

- There are two paths between switches

- This creates a loop

## Scenario: Ping with Wrong IP Address and Broadcasting in Switch Network

When PC1 tries to ping PC2 but enters the wrong IP address, the following process happens in the network:

- PC1 sends a ping request to the destination IP address.

- Before sending the packet, PC1 must find the destination MAC address.

- If the MAC address is unknown, PC1 sends an ARP Request, which is a broadcast message to the network.

- The switch receives this broadcast frame and forwards (floods) it to all ports within the same VLAN.

- If multiple switches are connected with redundant links (loops), the broadcast frame can circulate repeatedly between switches.

Because of this continuous broadcasting in a looped network, the following problems can occur:

## Problems Caused by Continuous Broadcasting

### 1) Broadcast Storm

A broadcast storm happens when broadcast packets keep circulating in the network without stopping.
This consumes bandwidth and network resources, causing the network to slow down or become unavailable.

### 2) MAC Table Instability (MAC Table Confusion)

Switches learn MAC addresses from incoming frames.
When frames loop continuously, the switch keeps updating the same MAC address on different ports.
This frequent change creates instability in the MAC address table.

### 3) Network Congestion or Network Failure

The excessive traffic generated by looping broadcasts can overload switches and links.
As a result, normal communication becomes very slow or the network may completely stop working.

# 3) How STP Uses Root Bridge to Prevent the Loop

STP solves the problem by selecting one main switch called the Root Bridge.

## Step-by-Step Process

### Step 1 — Elect Root Bridge

STP chooses:

Switch with the lowest Bridge ID

Bridge ID:

Priority + MAC Address

That switch becomes:

Root Bridge

### Step 2 — Find Best Path to Root

Every switch calculates:

Shortest path to the Root Bridge

That port becomes:

Root Port

### Step 3 — Block Extra Path

If there are multiple paths:

STP will:

Block one port

This removes the loop.

# 4) Why Root Bridge Is Important

The Root Bridge acts like the central reference point.

All switches:

- Send traffic toward the Root

- Use the shortest path

- Block extra paths

So the network becomes:

Loop-free

5) TOPOLOGY

## What the Colors Mean in Your Diagram

From your image:

- Green ports = Forwarding state (working normally)

- Orange port = Blocking state (disabled by STP to prevent loop)

You connected all switches in a ring topology, which creates multiple paths. STP must block one path to keep the network loop-free.

## 6) How to Check Which Switch is the Root Switch (in STP)

In Spanning Tree Protocol (STP) for Cisco Certified Network Associate (CCNA), you can easily identify the Root Switch using simple commands on the switch.

## Method 1 — Use the Main Command

en

Conf t

do show spanning-tree

### On the Root Switch, you will see:

This bridge is the root

That line confirms:

This switch is the Root Switch.