1.INTER-VLAN:

Normally, devices in different VLANs cannot communicate with each other. To enable communication between different VLANs, we need a method called Inter-VLAN routing.

## Why Do We Use Inter-VLAN Communication?

- To allow communication between devices in different VLANs (e.g., CS talking to Admin).

- Still maintain isolation and security for local traffic.

- Useful in large networks with multiple departments.

### 1. Layer 3 Switch Inter-VLAN Routing (Full Layer 3 Routing)

This method uses a Layer 3 switch to perform the routing between VLANs without needing a physical router. A Layer 3 switch is capable of routing traffic between VLANs using its SVI (Switched Virtual Interface).

### Layer 3 Switch Configuration Example

#### Network Design:

- VLAN 10: Computer Science

- VLAN 20: Electrical

- PCs:

  - PC1: 192.168.10.2 (VLAN 10)

  - PC2: 192.168.10.3 (VLAN 10)

  - PC3: 192.168.20.2 (VLAN 20)

  - PC4: 192.168.20.3 (VLAN 20)

FIRST GO TO ALL PC AND GIVE IP ADDRESS.

CLICK ON DEVICE – GO TO DESKTOP – IP CONFIGURATION.

AND PROVIDE IP AND SUBNET MASK AND GETWAY IP. AND DO SAME FOR ALL PCS.

#### Layer 3 Switch Configuration:

GO TO SWITCH AND CLICK ON CLI

CREATE VLAN.

Switch> enable

Switch# configure terminal

Switch(config)# vlan 10

Switch(config)# name Computer_Science

Switch(config)# vlan 20

Switch(config)# name Electrical

  Assign VLANs to Ports:

Switch(config)# interface range fa0/1 - 2

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 10

Switch(config)# interface range fa0/3 - 4

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 20

  Enable Routing on the Layer 3 Switch:

Switch(config)# ip routing

  Create SVI Interfaces for Each VLAN:

Switch(config)# interface vlan 10

Switch(config-if)# ip address 192.168.10.1 255.255.255.0

Switch(config-if)# no shutdown

What this does:

- Creates a virtual interface for VLAN 10

- Assigns a gateway IP address

- Enables the interface

Switch(config)# interface vlan 20

Switch(config-if)# ip address 192.168.20.1 255.255.255.0

Switch(config-if)# no shutdown

- NOW PC1 -DESKTOP- COMMAND PROMPT(192.168.10.2) can ping PC3 (192.168.20.2).

- AND PC2 -DESKTOP- COMMAND PROMPT (192.168.10.3) can ping PC4 (192.168.20.3).

# 2.What is ICMP?

ICMP (Internet Control Message Protocol) is a protocol used to test connectivity and send error messages between network devices.

### Simple definition:

ICMP is used to check whether a device is reachable on the network.

### Example:

When you run:

ping 192.168.20.10

The system sends:

- ICMP Echo Request → "Are you reachable?"

- ICMP Echo Reply → "Yes, I am reachable"

# 3.Where ICMP is used

Most common tools:

- ping → Uses ICMP

- tracert / traceroute → Uses ICMP

- Network troubleshooting

- Connectivity testing

So whenever you ping between VLANs, you are using ICMP.

# 4. Why ping fails the first time in Inter-VLAN (different VLAN)

This is normal behavior, and it usually happens because of ARP (Address Resolution Protocol).

## Root Cause: ARP Process

Before sending ICMP, the device must first learn the MAC address of the destination or gateway.

So the first ping is used to:

- Discover MAC address

- Build ARP table

- Then communication works